- Internal Blockage / Point of Use

These affect specific outlets rather than the entire villa.

A) Bathrooms - Showers :Showerheads blocked due to limescale buildup

Impact: Weak or inconsistent flow at specific showers only

B) Kitchen & Utility Areas: Blocked water filters

Impact: pressure drop at specific appliances or taps

- Pipe sizing Issue

Mechanism: Pressure loss related to pipe diameter, smaller pipes

Issues:

- Undersized pipes (common in older villas or cost-optimized builds)

- Long pipe runs with small diameters

Impact:

- Reduced flow rate at outlets

- Pressure drops when multiple taps are used

- How to detect : Good pressure in one tap, weak when multiple taps open

- Pipe Leaks

Water Pressure problem : Leaks cause pressure drop before water reaches outlets.

Issues:

- Underground leaks

- Wall leaks (not always visible)

Impact: Weak pressure across the outlets

- Increased water bills

How to detect :

- Sudden drop in water pressure in the whole villa (not just one tap)

- Pump running even when no water is being used

- Higher DEWA water bill than usual

- Hot Water System Issues

These apply only when hot water pressure is affected.

Key Causes: Residue buildup in water heater : Blocks hot water outlet

Water Pressure problem :

If Cold water ,normal pressure
if Hot water , weak pressure

- Airlocks in Pipes

Cause: Air trapped in pipes interrupts water flow.

Issues: After tank refill or maintenance

- Poor system sealing

Impact: Interrupted pressure or irregular flow

Diagnostic signal:  Sudden bursts of air + water from taps

- Single pipe network serving all floors without zoning

Cause:

- There is one main supply line delivering water to the whole villa

- There are no pressure zones, no separate pipelines, and no pressure regulators dedicated to specific floors.

- All taps and showers depend on the same pressure level.

- It’s harder to maintain consistent pressure for higher floors.

Impact: Uneven pressure distribution across the villa

How to Interpret the Problem

If pressure is low everywhere: Likely pump, tank, or supply issue

If pressure differs by floor: Likely structural or pipe distribution issue

If pressure drops when multiple taps are used: System limitation (most common in villas)

If pressure is low in one tap only: One point of Use issue

If only hot water is affected: Water heater issue